- What are these sections in the elf file?

These sections are named regions of the elf file that groups together a specific type of data, so the linker and loader know how to place it in memory and how to use it.

- What is the address of symbol kernel_main? What are the first 8 bytes at the symbol? What are the corresponding instructions?

00000000000809d8.

a9bf7bfd	stp	x29, x30, [sp, #-16]!

910003fd	mov	x29, sp

- How many bytes does each aarch64 instruction contain?

4 bytes

- How is kernel8.img generated out of kernel8.elf?

It is generated by extracting the raw loadable binary contents and removing the ELF metadata from the .elf file to make a flat binary image.

Using the command:
aarch64-linux-gnu-objcopy -O binary kernel8.elf kernel8.img

- Now examine kernel8.img (use the hexdump command or the VSCode plugin). Search for the first 8 bytes of kernel_main(). Can you find it? At which offset of kernel8.img?

Yes, the bytes fd 7b bf a9 fd 03 00 91 is in the .img file. The offset is 0x9d8.